PS 3: Teleoperated Robot Using ROS

Technical Report

ROS 2 Jazzy | ESP32 | WSL2 Ubuntu 24.04

# 1. Project Overview

This project implements a teleoperated differential drive robot controlled entirely through ROS 2 Jazzy running on WSL2 Ubuntu 24.04. The robot uses an ESP32 microcontroller as its onboard brain, communicating with the ROS system over WiFi. An operator controls the robot using keyboard arrow keys, and the system supports variable speed control, path recording, and autonomous path retracing.

## 1.1 Hardware Components

| Component | Role | Details |
| --- | --- | --- |
| ESP32 Dev Module | Robot brain / WiFi AP | Runs motor logic, hosts TCP server on port 8080 |
| L298N Motor Driver | Motor control | Controls 2 DC motors via IN1-IN4 and ENA/ENB pins |
| 2x DC Motors | Differential drive | Left and Right drive wheels |
| Caster Wheel | Balance | Passive third contact point |
| 12V Battery | Power source | Powers motors via L298N |
| Buck Converter 12V to 5V | ESP32 power | Steps down battery voltage for ESP32 |
| Smartphone | Vision sensor | Streams live video via IP Webcam app |

## 1.2 Software Stack

| Software | Version | Purpose |
| --- | --- | --- |
| ROS 2 | Jazzy | Robot middleware and communication |
| WSL2 | Ubuntu 24.04 | Linux environment on Windows |
| Arduino IDE | 2.x | ESP32 firmware development |
| Python | 3.12 | ROS nodes (bridge, teleop, camera) |
| OpenCV | 4.x | Camera frame processing |
| IP Webcam | Android App | Phone camera streaming |

# 2. System Architecture

The system is built as a layered pipeline where each component has a single responsibility. The operator interacts only with the teleop node, and commands flow through ROS topics to the bridge node, which translates them into TCP packets for the ESP32.

## 2.1 Communication Pipeline

Keyboard Input

|

v

arrow_teleop node   (ROS 2 Node)

|  publishes Twist msg

v

/cmd_vel topic      (ROS 2 Topic)

|

v

esp32_bridge node   (ROS 2 Node)

|  TCP Socket over WiFi

v

ESP32 (192.168.4.1:8080)

|

v

L298N Motor Driver

|

v

DC Motors (Left + Right)

## 2.2 WiFi Network

The ESP32 operates as a WiFi Access Point (AP) with the following credentials:

- SSID: ESP_ROBOT

- Password: 12345678

- ESP32 IP: 192.168.4.1 (default softAP IP)

- TCP Server Port: 8080

Both the laptop running ROS 2 and the smartphone camera connect to this hotspot. WSL2 uses mirrored networking mode (configured in .wslconfig) so it shares the Windows host network interface and can directly reach 192.168.4.1.

## 2.3 Packet Format

All communication between the ROS bridge and ESP32 uses a simple comma-separated text protocol over TCP:

lin,ang,speed,flag\n

| Field | Type | Description | Example |
| --- | --- | --- | --- |
| lin | float | Linear velocity. +1.0 = forward, -1.0 = backward | 1.000 |
| ang | float | Angular velocity. +1.0 = left, -1.0 = right | -1.000 |
| speed | int (0-8) | Speed level from teleop | 4 |
| flag | string | RETRACE to trigger path retracing, else empty | RETRACE |

Example packets:

1.000,0.000,4,          <- Forward at speed level 4

-1.000,0.000,6,         <- Backward at speed level 6

0.000,1.000,4,          <- Turn left at speed level 4

0.000,0.000,0,RETRACE   <- Trigger path retrace

# 3. ROS 2 Package Setup

## 3.1 Creating the Workspace

A standard ROS 2 Python package was created inside a colcon workspace:

mkdir -p ~/ros2_ws/src

cd ~/ros2_ws/src

ros2 pkg create esp32_robot --build-type ament_python --dependencies rclpy geometry_msgs

## 3.2 Package Structure

ros2_ws/

src/

esp32_robot/

esp32_robot/

__init__.py

esp32_bridge.py     <- WiFi bridge node

arrow_teleop.py     <- Keyboard teleop node

camera_publisher.py <- Phone camera node

setup.py              <- Package config

package.xml           <- Dependencies

launch/

robot.launch.py     <- Launch file

## 3.3 setup.py Entry Points

Each Python file is registered as an executable ROS node in setup.py:

entry_points={

'console_scripts': [

'esp32_bridge     = esp32_robot.esp32_bridge:main',

'arrow_teleop     = esp32_robot.arrow_teleop:main',

'camera_publisher = esp32_robot.camera_publisher:main',

],

},

## 3.4 Building the Package

cd ~/ros2_ws

colcon build --packages-select esp32_robot

source install/setup.bash

# 4. ESP32 Firmware (robot_esp32.ino)

The ESP32 firmware handles WiFi connectivity, motor control, speed management, move stack recording, and autonomous path retracing. It runs a TCP server that accepts one client connection at a time.

## 4.1 Pin Configuration

| Pin | GPIO | Function |
| --- | --- | --- |
| IN1 (Motor A) | GPIO 14 | Left motor direction pin 1 |
| IN2 (Motor A) | GPIO 27 | Left motor direction pin 2 |
| IN3 (Motor B) | GPIO 32 | Right motor direction pin 1 |
| IN4 (Motor B) | GPIO 19 | Right motor direction pin 2 |
| ENA | GPIO 12 | Left motor PWM enable |
| ENB | GPIO 33 | Right motor PWM enable |

Note: GPIO 25 and 26 are DAC (Digital to Analog Converter) pins on ESP32 and were found to be unreliable for digital motor control. All direction pins were moved to safe GPIO pins with no alternate functions.

## 4.2 PWM Speed Control

The ESP32 Arduino core 3.x uses ledcAttach for PWM generation on the enable pins. The speed level sent from ROS (0 to 8) is mapped to PWM duty cycle (0 to 255):

ledcAttach(en1, 1000, 8);   // 1kHz, 8-bit resolution

ledcAttach(en2, 1000, 8);

currentPWM = map(speedLevel, 0, 8, 0, 255);

ledcWrite(en1, currentPWM);

ledcWrite(en2, currentPWM);

## 4.3 Motor Direction Logic

The left motor (Motor A) is physically wired in reverse compared to the right motor (Motor B). Instead of rewiring, the firmware compensates by inverting the pin logic for Motor A:

| Command | Motor A (Left) IN1/IN2 | Motor B (Right) IN3/IN4 | Result |
| --- | --- | --- | --- |
| F (Forward) | LOW / HIGH (inverted) | HIGH / LOW (normal) | Both wheels forward |
| B (Backward) | HIGH / LOW (inverted) | LOW / HIGH (normal) | Both wheels backward |
| L (Left turn) | HIGH / LOW (inverted) | HIGH / LOW (normal) | Left back, Right forward |
| R (Right turn) | LOW / HIGH (inverted) | LOW / HIGH (normal) | Left forward, Right back |

## 4.4 Move Stack and Path Recording

Every movement command is recorded in a stack (array) along with its duration. This enables the retrace feature to replay movements in reverse order:

struct Move {

char          cmd;       // 'F', 'B', 'L', 'R'

unsigned long duration;  // milliseconds

};

Move stack[100];

When the command changes (e.g. from F to L), the previous command and how long it ran is pushed onto the stack. The stack can hold up to 100 moves.

## 4.5 Path Retracing

When a RETRACE command is received, the firmware replays all recorded moves in reverse order, executing the opposite command for the same duration:

F for 3000ms  ->  stored as (F, 3000)

L for 1500ms  ->  stored as (L, 1500)

RETRACE triggers:

Step 1: R for 1500ms   (reverse of L)

Step 2: B for 3000ms   (reverse of F)

-> Robot returns to starting position

Retracing is non-blocking. The updateRetrace() function is called every loop() iteration and checks if the current step's duration has elapsed before advancing to the next step. Any movement command received during retracing immediately interrupts it and clears the stack.

## 4.6 WiFi TCP Server

The ESP32 creates a WiFi Access Point and runs a TCP server. It accepts one client at a time. When the client disconnects, motors are stopped automatically:

WiFi.softAP("ESP_ROBOT", "12345678");

server.begin();  // port 8080

// In loop():

client = server.available();

String data = client.readStringUntil('\n');

# 5. ROS Bridge Node (esp32_bridge.py)

The bridge node is the critical link between the ROS ecosystem and the physical robot. It subscribes to the /cmd_vel topic, translates Twist messages into TCP packets, and manages the WiFi socket connection to the ESP32.

## 5.1 Subscriptions and Topics

| Topic | Message Type | Direction | Purpose |
| --- | --- | --- | --- |
| /cmd_vel | geometry_msgs/Twist | Subscribe | Receives velocity and retrace commands |

## 5.2 Twist Message Field Usage

The standard ROS Twist message is used creatively to carry extra information beyond just velocity:

| Twist Field | Value Used | Meaning |
| --- | --- | --- |
| linear.x | +1.0 / -1.0 / 0.0 | Forward / Backward / None |
| angular.z | +1.0 / -1.0 / 0.0 | Left / Right / None |
| linear.y | 0 to 8 | Speed level (piggybacked) |
| linear.z | 1.0 or 0.0 | Retrace flag (1.0 = retrace) |

## 5.3 Retrace Suppression Logic

A key challenge was that when the retrace key is pressed, the teleop node sends a retrace command followed immediately by a stop command (since linear and angular are both zero). Without suppression, this stop command would arrive at the ESP32 and interrupt the retrace immediately after starting it.

The bridge solves this with a self.retracing flag:

if retrace:

self.retracing = True

send RETRACE packet to ESP32

return

if self.retracing and lin == 0 and ang == 0:

# suppress this stop packet

return

if self.retracing and (lin != 0 or ang != 0):

# real movement key pressed - interrupt

self.retracing = False

## 5.4 Connection Reliability

The bridge includes a watchdog timer that runs every 2 seconds and automatically reconnects to the ESP32 if the connection is lost. All socket operations use a threading lock to prevent race conditions between the watchdog and the callback:

self.create_timer(2.0, self.watchdog)

def watchdog(self):

if not self.connected:

self.connect()

# 6. Teleop Node (arrow_teleop.py)

The teleop node captures raw keyboard input and publishes Twist messages to the /cmd_vel topic. It uses the termios and tty Python libraries to read single keystrokes in raw mode without requiring the Enter key.

## 6.1 Key Bindings

| Key | Action | linear.x | angular.z | linear.z |
| --- | --- | --- | --- | --- |
| Arrow Up | Forward | +1.0 | 0.0 | 0.0 |
| Arrow Down | Backward | -1.0 | 0.0 | 0.0 |
| Arrow Left | Turn Left | 0.0 | +1.0 | 0.0 |
| Arrow Right | Turn Right | 0.0 | -1.0 | 0.0 |
| Space | Stop | 0.0 | 0.0 | 0.0 |
| r | Retrace path | 0.0 | 0.0 | 1.0 |
| + | Speed up (2 levels) | 0.0 | 0.0 | 0.0 |
| - | Speed down (2 levels) | 0.0 | 0.0 | 0.0 |
| q / Ctrl+C | Quit and stop | 0.0 | 0.0 | 0.0 |

## 6.2 Arrow Key Detection

Arrow keys send 3-byte escape sequences. The node reads the first byte and if it is ESC (\x1b), reads 2 more bytes to form the full sequence:

Arrow Up    = \x1b[A

Arrow Down  = \x1b[B

Arrow Right = \x1b[C

Arrow Left  = \x1b[D

## 6.3 Speed Control

Speed is stored as a level from 0 to 8. Each press of + or - changes the level by 2. The level is sent in the linear.y field of the Twist message and mapped to PWM on the ESP32:

| Speed Level | PWM Value | Percentage |
| --- | --- | --- |
| 0 | 0 | 0% |
| 2 | 64 | 25% |
| 4 (default) | 127 | 50% |
| 6 | 191 | 75% |
| 8 | 255 | 100% |

# 7. Vision System - Phone Camera

Instead of adding a dedicated onboard camera module, a smartphone running the IP Webcam app was used as a wireless vision sensor. This approach provided higher image quality and simpler setup while still being fully integrated into the ROS pipeline.

## 7.1 Architecture

Smartphone Camera

|

v

IP Webcam App (MJPEG stream over HTTP)

|  http://phone_ip:8080/video

v

camera_publisher.py node (OpenCV capture)

|  cv_bridge conversion

v

/camera/image_raw topic (sensor_msgs/Image)

|

v

rqt_image_view (live FPV display)

## 7.2 Common WiFi Network

The ESP32 Access Point serves as the common wireless network for both the robot control and the camera feed. The laptop connects to ESP_ROBOT hotspot, and the smartphone also connects to ESP_ROBOT. This means the robot control packets and the camera stream share the same WiFi network, keeping the system self-contained without needing an external router.

## 7.3 Camera Publisher Node

The camera_publisher.py node uses OpenCV's VideoCapture to read MJPEG frames from the phone stream and publishes them as ROS Image messages at 15 FPS:

STREAM_URL = "http://phone_ip:8080/video"

cap = cv2.VideoCapture(STREAM_URL)

ret, frame = cap.read()

msg = bridge.cv2_to_imgmsg(frame, encoding='bgr8')

publisher.publish(msg)

## 7.4 Viewing the Feed

The live video feed is viewed using rqt_image_view, a standard ROS 2 GUI tool:

ros2 run rqt_image_view rqt_image_view

The operator selects /camera/image_raw from the dropdown and sees real-time FPV video while controlling the robot with arrow keys.

## 7.5 Why Phone Camera Over ESP32-CAM

| Aspect | Phone Camera | ESP32-CAM |
| --- | --- | --- |
| Image Quality | High (720p-1080p) | Low (VGA) |
| Setup Complexity | Low (install app) | High (extra hardware, code) |
| Hardware Changes | None to robot | Requires wiring |
| Streaming Latency | Low over WiFi | Low but compressed |
| Integration | Standard ROS topic | Needs custom bridge |

# 8. Running the Complete System

## 8.1 Prerequisites

- ESP32 flashed with robot firmware

- Laptop WiFi connected to ESP_ROBOT hotspot

- WSL2 running with networkingMode=mirrored in .wslconfig

- Phone running IP Webcam app and connected to ESP_ROBOT

- ROS 2 Jazzy installed and workspace built

## 8.2 Launch Sequence

Open 4 separate WSL terminals:

Terminal 1 - Bridge Node:

source ~/ros2_ws/install/setup.bash

ros2 run esp32_robot esp32_bridge

Terminal 2 - Teleop Node:

source ~/ros2_ws/install/setup.bash

ros2 run esp32_robot arrow_teleop

Terminal 3 - Camera Node:

source ~/ros2_ws/install/setup.bash

ros2 run esp32_robot camera_publisher

Terminal 4 - Camera Viewer:

ros2 run rqt_image_view rqt_image_view

## 8.3 Verification Commands

ros2 node list

# Expected: /esp32_bridge  /arrow_teleop  /camera_publisher

ros2 topic list

# Expected: /cmd_vel  /camera/image_raw

ros2 topic echo /cmd_vel

# Press arrow keys and verify values appear

# 9. Features Summary

| Feature | Implementation | Status |
| --- | --- | --- |
| Forward / Backward | Arrow Up/Down -> Twist linear.x -> ESP32 F/B | Complete |
| Left / Right | Arrow Left/Right -> Twist angular.z -> ESP32 L/R | Complete |
| Speed Control | 8 levels via +/- keys, PWM mapped 0-255 | Complete |
| 4-Wheel Differential Drive | 2 drive motors + caster, independent PWM control | Complete |
| Path Retracing | Move stack records all moves, r key triggers reverse replay | Complete |
| Retrace Interrupt | Any movement key during retrace stops and clears it | Complete |
| Live Camera Feed | Phone stream -> ROS topic -> rqt_image_view | Complete |
| Auto Reconnect | Bridge watchdog timer reconnects on ESP32 disconnect | Complete |
| Motor Safety | Motors stop on client disconnect automatically | Complete |

# 10. WSL2 Networking Configuration

Running ROS 2 inside WSL2 introduced a networking challenge. By default WSL2 uses NAT networking which isolates it from the Windows host network. This means the ESP32 WiFi hotspot (connected on Windows) would not be reachable from inside WSL2.

The solution was to enable mirrored networking in WSL2, which makes WSL2 share the same network interfaces as Windows:

File: C:\Users\username\.wslconfig

[wsl2]

networkingMode=mirrored

After restarting WSL2 with wsl --shutdown, the ESP32 at 192.168.4.1 became directly reachable from inside WSL2, verified with:

ping 192.168.4.1

# 64 bytes from 192.168.4.1: icmp_seq=1 ttl=64 time=3.06ms